# PFC2D Darcy Flow and Fracture Simulation Scripts – Documentation

## 1. Project Files and Call Structure

This PFC2D project consists of a set of .p2dat scripts for building a bonded-particle rock sample, applying Darcy-type flow, and tracking fracture evolution. The main scripts and their roles are summarized below.

| File name | Main function |
| --- | --- |
| DOALL.p2dat | Master control script. Calls the geometry, contact, rock-state and cavity scripts in sequence. |
| 01-SET.p2dat | Geometry and particle generation: model size, particle radii, porosity and initial packing. |
| 02-Contact.p2dat | Contact models and micro-parameters: stiffness, strength and friction for different materials. |
| 03-Rongshi.p2dat | Rock-state saving: stores a consistent “base” bonded-rock sample. |
| 04-Dakong.p2dat | Drilling / cavity creation near the sample centre for flow boundary conditions. |
| 05-Daxi.p2dat | Darcy flow loading and calculation: flow boundary, cycling and history recording. |
| crack_num.p2dat | Fracture statistics: counts bond-break events (tension vs shear, different bond types). |
| fracture.p2dat | Floater elimination and packing stabilization before flow and fracturing. |
| dom.p2dat | Domain (flow region) identification and outer-boundary detection. |
| dom1.p2dat | Pressure-field update and volume balance for coupled flow calculations. |

In practice, DOALL.p2dat is first executed to build and save the base model with a central cavity. Then 05-Daxi.p2dat is run to perform the Darcy flow and fracture evolution simulation.

## 2. Geometry and Particle Generation (01-SET.p2dat)

01-SET.p2dat defines the model dimensions, particle properties and the initial packing.

1) Model dimensions and coordinates: A characteristic length L1 = 50 mm is used. The sample width and height are 0.5·L1 (approximately 25 mm × 25 mm). Upper, lower, left and right boundaries are defined accordingly. Ball_porosity, radius_min and radius_max control the initial particle size distribution and target porosity.

2) Particle generation and compaction: The particles are generated and compacted using gravity and damping (a_damp) until a dense packing is obtained. The floater-elimination logic in fracture.p2dat is called to remove particles with too few contacts, which improves the mechanical stability of the assembly.

3) Particle grouping: ball group commands are used to assign certain regions to specific groups (for example, a central “filling” group), while the remaining particles are treated as the rock matrix. This allows different contact properties or local modifications to be applied in later steps.

## 3. Contact Models and Micro-Parameters (02-Contact.p2dat)

02-Contact.p2dat assigns stiffness, strength and friction parameters to the particle assembly and sets up the relevant contact models (parallel bonds, flatjoints, smoothjoints, etc.).

1) Material parameters: For different materials (e.g., metamorphic rock, sandstone), the script defines contact Young's modulus (emod), normal-to-shear stiffness ratio (kratio), tensile strength (ten), cohesion, friction angle (fa), bond gap and friction coefficient (fric).

2) Contact and bond models: Using cmat and contact method bond, the script assigns linear, parallel-bond, flatjoint and smoothjoint models to ball–ball and ball–wall contacts, and maps the above parameters to each material combination.

3) State saving: After assigning the contact parameters, the model is saved as 02-FJ-SJ-bond_Add, which is then used by 03-Rongshi.p2dat as the base bonded-rock sample.

## 4. Rock-State Saving and Cavity Creation (03-Rongshi.p2dat and 04-Dakong.p2dat)

1) 03-Rongshi.p2dat: Restores 02-FJ-SJ-bond_Add and saves it as 03-Rongshi. This file represents the reference bonded-rock sample before introducing flow-specific modifications.

2) 04-Dakong.p2dat: Restores 03-Rongshi and deletes particles within a given radius around the sample centre. This operation drills a cavity or “wellbore” that later serves as a flow boundary. The updated model is saved as 04-Dakong.

## 5. Darcy Flow Loading and Boundary Conditions (05-Daxi.p2dat)

05-Daxi.p2dat is the core Darcy-flow script. It applies flow boundaries, controls loading, and records histories.

1) Model restore and wall reset: The script restores the 04-Dakong state and deletes existing walls, preparing for the application of new boundary conditions.

2) Loading and strength-related parameters: Using loading_rate and related coefficients (xishu_e, xishu_s), the script sets upper bounds for stiffness and strength (emod_max, pb_emod_max, pb_ten_m, pb_ten_c) so that different experimental conditions can be matched.

3) Flow boundary and time-step settings: A function such as flow_bc_circle is called to apply a flow boundary at the central cavity. The gap_mul parameter adjusts the relation between fracture aperture and flow rate. The script defines the time step, mechanical time and several histories (e.g., time, flow-related variables, and crack_num), runs a specified number of cycles, and finally saves the model as darcy_test.

## 6. Domain Identification and Pressure Update (dom.p2dat and dom1.p2dat)

dom.p2dat and dom1.p2dat form a FISH toolkit for flow-domain identification and pressure/volume updates.

1) dom.p2dat: By scanning all contacts, the script builds connectivity information and groups particles that are connected into “domains”. The domain with the largest number of particles is treated as the outer domain, representing the external or infinite boundary region.

2) dom1.p2dat: For each domain, a pressure is defined and updated. For each contact between domains, the code calculates a pipe flow rate based on the pressure difference, an effective permeability factor and the contact aperture. The associated volume changes are accumulated for the two domains to enforce global volume balance. Pressure gradients are then converted into equivalent forces acting on the particles, enabling fully coupled fluid–solid interaction.

## 7. Fracture Statistics and Floater Removal (crack_num.p2dat and fracture.p2dat)

1) crack_num.p2dat: This script tracks bond-break events in BPM/CBM/FJM/SJM contacts during the simulation. It identifies the contact model (linearcbond, linearpbond, flatjoint, smoothjoint, etc.), distinguishes between tensile and shear failures, and accumulates separate counters for each type. The fragment and ball-result logic is used to record fragment IDs. Global variables such as crack_num are exposed so that they can be stored as histories in 05-Daxi.p2dat.

2) fracture.p2dat: Despite its name, this script mainly implements the floater-elimination algorithm. Functions such as flt_eliminate and flt_num identify particles with fewer than a specified number of contacts and adjust or remove them. Internal counters track the number of floating particles, active particles and contact forces. This procedure produces a denser, mechanically more realistic packing and reduces numerical noise in later flow and fracture simulations.

## 8. Usage Notes and Recommendations

1) Recommended execution sequence

Step 1: Run DOALL.p2dat to build the geometry, assign contacts, save the rock state and drill the central cavity (ending with the 04-Dakong state).

Step 2: Run 05-Daxi.p2dat to apply flow boundary conditions, perform Darcy flow and fracture evolution, and save the final darcy_test state along with history curves.

2) Micro-parameter tuning

To match different confining stresses or loading paths, adjust emod, cohesion and ten in 02-Contact.p2dat. To promote more tensile or shear fracture, reduce the corresponding strength parameters. Flow sensitivity can be controlled using the permeability factors in dom1.p2dat and parameters such as gap_mul in 05-Daxi.p2dat.

3) Post-processing

Histories typically include time, flow-related quantities (e.g., cumulative flow, pressure difference) and the number of fractures. It is recommended to export these data to text or spreadsheet files and analyze them with Python or MATLAB, for example to obtain permeability–time curves, fracture-count–strain curves, and the evolving proportion of tensile versus shear fractures.

4) Use as project and paper documentation

This English Word document can serve as both the user manual for the PFC2D Darcy-flow and fracture case and, after further polishing, as the basis for a “Numerical Model and Methods” section in a journal manuscript.

## Appendix: Full .p2dat Script Listings

The following sections list the complete contents of all .p2dat files included in this project, for reference and archiving.

### DOALL.p2dat

new

call 01-SET.p2dat

call 02-Contact.p2dat

call 03-Rongshi.p2dat

call 04-Dakong.p2dat

return

### 01-SET.p2dat

new

domain extent -2.6 2.6 condition destroy

def setup

L1=50e-3

height=L1*0.5

height_=-L1*0.5

width_left=-L1*0.5

width_right=L1*0.5

fc=0.0

a_damp=0.1

p_density=2000

Ball_porosity = 0.08

radius_min= 0.004*L1

radius_max=[radius_min]*1.66

end

@setup

set random 1087;

wall generate box [-0.5*L1] [0.5*L1] [-0.5*L1] [0.5*L1] ...

expand 1.5

wall property kn=1e11 ks=0.0 fric 0.0

ball distribute porosity [Ball_porosity]  radius [radius_min]  [radius_max] ...

box [width_left] [width_right] [height_] [height] ...

group 'rock'

ball attribute density 2700.0 damp 0.7 ;range group 'rock'

cmat default model linear method deform emod 100e6 kratio 1.5 ...

property fric 0.0 dp_nratio 0.0 dp_sratio 0.0

;set timestep scale 2

cycle 5000 calm 100

;ball delete range y -100 @height_

;ball delete range y  @height 100

set timestep auto

solve aratio 1e-5

calm

call fracture.p2dat

[flt_remain = 0.0]

[flt_def = 3]

@flt_eliminate

ball attribute displacement multiply 0.0 rotation multiply 0.0 ...

velocity multiply 0.0 spin multiply 0.0

calm

save 00-int

;ball group 'rock'

ball group 'tianchong' range ...

cylinder end1 -0.0125 -0.0125 end2 0.0125  0.0125  radius 0.001

;ball group 'rock' range not group 'tianchong'

save 01-SET

### 02-Contact.p2dat

restore 01-SET

def contact_parameters

rock_emod_Contact=5.0e9

rock_kratio_Contact=0.61

rock_ten_Contact=3.07e6

rock_cohesion_Contact=81.00e7;81.00e6

rock_fa_Contact=26.56505118

rock_gap_contact=0.004*L1

rock_fric_Contact=0.5

sandstone_emod_Contact=4.15e9

sandstone_kratio_Contact=0.95

sandstone_ten_Contact=2.54e6

sandstone_cohesion_Contact=34.76e6

sandstone_fa_Contact=26.56505118

sandstone_gap_contact=0.004*L1

sandstone_fric_Contact=0.5

Dfn_kn_sj=280e9

Dfn_ks_sj=260e9

Dfn_fric_sj=0.7

Dfn_fa_sj=60

Dfn_coh_sj=4.3e6

Dfn_ten_sj=5.2e6

Dfn_large_sj=1

Dfn_gap_contact=0.004*L1

end

@contact_parameters

contact groupbehavior and

cmat default type ball-ball ...

model smoothjoint ...

property sj_kn [Dfn_kn_sj] ...

sj_ks [Dfn_ks_sj] ...

sj_fric [Dfn_fric_sj] ...

sj_coh [Dfn_coh_sj] ...

sj_ten [Dfn_ten_sj] ...

sj_large [Dfn_large_sj]

contact method bond gap [Dfn_gap_contact]

cmat default type ball-facet ...

model  linear ...

method deform emod [rock_emod_Contact] kratio [rock_kratio_Contact] ...

property fric 0.0 dp_nratio 0.0 dp_sratio 0.0

cmat add 1   model flatjoint ...

method deform emod [rock_emod_Contact] ...

kratio [rock_kratio_Contact] ...

method bond gap [rock_gap_contact] ...

property fj_fric [rock_fric_Contact]  ...

fj_fa [rock_fa_Contact] ...

fj_coh [rock_cohesion_Contact] ...

fj_ten [rock_ten_Contact] ...

range group 'rock'

cmat apply

cmat add 2   model flatjoint ...

method deform emod [sandstone_emod_Contact] ...

kratio [sandstone_kratio_Contact] ...

method bond gap 0.0 ...

property fj_fric 0.0  ...

fj_fa [sandstone_fa_Contact] ...

fj_coh [sandstone_cohesion_Contact] ...

fj_ten [sandstone_ten_Contact] ...

range group 'tianchong'

cmat apply

contact property lin_force 0.0 0.0 lin_mode 1

ball attribute contactforce multiply 0.0 contactmoment multiply 0.0

cycle 1

solve aratio 1e-5 max_cycles 1000000

cycle 1

solve aratio 1e-5 max_cycles 1000000

save 02-FJ-SJ-bond_Add

return

### 03-Rongshi.p2dat

restore 02-FJ-SJ-bond_Add

save 03-Rongshi

### 04-Dakong.p2dat

restore 03-Rongshi

ball delete range annulus center 0 0 radius 0.0 0.004

save 04-Dakong

### 05-Daxi.p2dat

; "Darcy" flow test  - FISH formulation

;call soft_servo

restore 04-Dakong

wall delete walls

def basic_parameters1

loading_rate=1e-5

xishu_e=(2.7333*math.log(loading_rate/1e-5)+24.14)/17.114

xishu_s=(14.317*math.log(loading_rate/1e-5)+139.37)/133.09

emod_max=50e9

pb_emod_max=50e9

pb_ten_m=9e7

pb_ten_c=3e7

end

@basic_parameters1

call crack_num.p2dat

@track_init

call dom

call dom1

;set fish callback  9.0 @flow_run

set @ap_zero=1.0 @perm=0.1 @gap_mul=0.0 @fap_zero=1e8

set @bulk_w=100.  @flow_dt=0.001

@flow_props

ball group 'boundary_balls' range x -0.027 -0.023

ball group 'boundary_balls' range x 0.023 0.027

ball group 'boundary_balls' range y -0.027 -0.023

ball group 'boundary_balls' range y 0.023 0.027

ball attribute velocity multiply 0.0

ball fix velocity rang group boundary_balls

;set @flow_set=1 @x1_bc=-1.0 @x2_bc=1.0 @y1_bc=0.47 @y2_bc=0.55

;@flow_bc

;set @flow_set=1

;set @flow_set=5

set @flow_set=1

@flow_bc_circle(0.0,0.0,0.004)

set @flow_set=3 @p_given=15e6

@flow_bc_circle(0.0,0.0,0.004)

set @gap_mul=0.2

set timestep max 1e-6

set mech age 0.0

[time111=mech.age]

hist id 1 @time111

hist id 2 @dddtime

hist id 3 @crack_num

;hist id 3 @time_now

plot create plot 'aaa'

plot add hist 1 2

cycle 40000

save darcy_test

### crack_num.p2dat

; fname: crack_num.p2fis

;

; Simple environment to track fragmentation in a BPM,CBM,FJM,SJM(PFC 2D).

; Track bond_break events from linearcbond, linearpbond, flatjoint, and smoothjoint models, and record them as fractures.

; Use Fragment logic and Ball Result logic to record fragment IDs.

;==============================================================================

define add_crack(entries)

local contact = entries(1)

contact_model = contact.model(contact)

if contact_model = 'flatjoint' then

mode      = entries(3)

else

mode      = entries(2)

endif

local frac_pos   = contact.pos(contact)

local norm       = contact.normal(contact)

local dfn_label  = 'crack'

local frac_size

local bp1 = contact.end1(contact)

local bp2 = contact.end2(contact)

local ret = math.min(ball.radius(bp1),ball.radius(bp2));contact.method(contact,'pb_radius')

frac_size = ret

local inDir = vector(-comp.y(norm),comp.x(norm))

local vert1 = frac_pos + inDir * frac_size

local vert2 = frac_pos - inDir * frac_size

local arg = array.create(4)

arg(1) = 'vertices'

arg(2) = 2

arg(3) = vert1

arg(4) = vert2

;crack_num = crack_num + 1

if mode = 1 then

; failed in tension

;dfn_label = dfn_label + '_tension'

;crack_tension_num = crack_tension_num + 1

dfn_label = 'crack_tension' + '_' + contact_model

if contact_model = 'linearcbond' then

crack_tension_num_linearcbond = crack_tension_num_linearcbond + 1

else if contact_model = 'linearpbond' then

crack_tension_num_linearpbond = crack_tension_num_linearpbond + 1

else if contact_model = 'flatjoint' then

crack_tension_num_flatjoint = crack_tension_num_flatjoint + 1

else if contact_model = 'smoothjoint' then

crack_tension_num_smoothjoint = crack_tension_num_smoothjoint + 1

endif

crack_tension_num =   crack_tension_num_linearcbond ...

+ crack_tension_num_linearpbond ...

+ crack_tension_num_flatjoint ...

+ crack_tension_num_smoothjoint

else if mode = 2 then

; failed in shear

;dfn_label = dfn_label + '_shear'

;crack_shear_num = crack_shear_num + 1

dfn_label = 'crack_shear' + '_' + contact_model

if contact_model = 'linearcbond' then

crack_shear_num_linearcbond = crack_shear_num_linearcbond + 1

else if contact_model = 'linearpbond' then

crack_shear_num_linearpbond = crack_shear_num_linearpbond + 1

else if contact_model = 'flatjoint' then

crack_shear_num_flatjoint = crack_shear_num_flatjoint + 1

else if contact_model = 'smoothjoint' then

crack_shear_num_smoothjoint = crack_shear_num_smoothjoint + 1

endif

crack_shear_num =   crack_shear_num_linearcbond ...

+ crack_shear_num_linearpbond ...

+ crack_shear_num_flatjoint ...

+ crack_shear_num_smoothjoint

endif

crack_num = crack_tension_num + crack_shear_num

global dfn = dfn.find(dfn_label)

if dfn = null then

dfn = dfn.add(0,dfn_label)

endif

local fnew = dfn.addfracture(dfn,arg)

dfn.fracture.prop(fnew,'failure_strength') = strength

dfn.fracture.prop(fnew,'failure_age')  = mech.age

dfn.fracture.extra(fnew,1) = bp1

dfn.fracture.extra(fnew,2) = bp2

if crack_num == 1

fissureFirst = dfn.fracture.find(1)

fissureEnd1 = dfn.fracture.extra(fissureFirst,1)

fissureEnd2 = dfn.fracture.extra(fissureFirst,2)

fissureNorm = dfn.fracture.normal(fissureFirst)

;fissure_Initiation_Angle =  user.vector.dip(fissureFirst)

endif

crack_accum += 1

if crack_accum > 50

if frag_time < mech.age

frag_time = mech.age

crack_accum = 0

command

fragment compute

endcommand

; go through and update the fracture positions

;loop for (local i = 0, i < 2, i = i + 1)

;    name = 'crack_tension'

;    if i = 1

;        name = 'crack_shear'

;    endif

;    dfn = dfn.find(name)

loop foreach dfn dfn.list

if dfn # null

dfn_name = dfn.name(dfn)

dfn_name_head = string.sub(dfn_name, 1, 5)

if dfn_name_head = 'crack' then

loop foreach local frac dfn.fracturelist(dfn)

local ball1 = dfn.fracture.extra(frac,1)

local ball2 = dfn.fracture.extra(frac,2)

if ball1 # null

if ball2 # null

local len = dfn.fracture.len(frac)/2.0

local pos = (ball.pos(ball1)+ball.pos(ball2))/2.0

if comp.x(pos)-len > xmin

if comp.x(pos)+len < xmax

if comp.y(pos)-len > ymin

if comp.y(pos)+len < ymax

dfn.fracture.pos(frac) = pos

endif

endif

endif

endif

endif

endif

endloop

endif

endif

endloop

endif

endif

end

define track_init

;loop foreach local dfn dfn.list

;    dfn.delete(dfn)

;endloop

command

;dfn delete

ball result clear

fragment clear

fragment register ball-ball

endcommand

; activate fishcalls

command

;set fish callback bond_break remove @add_crack

set fish callback bond_break @add_crack

endcommand

; reset global variables

global crack_accum = 0

global crack_num = 0

global crack_tension_num = 0

global crack_shear_num = 0

global crack_tension_num_linearcbond = 0

global crack_tension_num_linearpbond = 0

global crack_tension_num_flatjoint   = 0

global crack_tension_num_smoothjoint = 0

global crack_shear_num_linearcbond = 0

global crack_shear_num_linearpbond = 0

global crack_shear_num_flatjoint   = 0

global crack_shear_num_smoothjoint = 0

global track_time0 = mech.age

global frag_time = mech.age

global xmin = domain.min.x()

global ymin = domain.min.y()

global xmax = domain.max.x()

global ymax = domain.max.y()

end

;==============================================================================

; eof: fracture.p3fis

### fracture.p2dat

; Filename: flt.FIS

;

; PURPOSE:  These functions used when attempting to eliminate

;           "floaters" (particles with less than [flt_def] contacts)

;           from the initial packed assembly.

;

; Itasca Consulting Group, Inc.

; ===================================================================

;

; ===================================================================

; PUBLIC FUNCTIONS

; ===================================================================

;

def flt_eliminate

;

; ----- Entry-level call to invoke floater-reduction algorithm.

;

; INPUT: flt_def    - balls with < [flt_def] contacts are "floaters"

;        flt_remain - proportion of total balls that can be floaters

;

_debug = 1  ; always dump run-tracking info. to screen

pc_zap_floaters

end

; ===================================================================

def pc_zap_floaters

;

; ----- Eliminate the floaters in the assembly.

;

;    CAUTION:  Friction will be set to zero! ... re-specify at end

;              Ball color indices will be changed!

;              All balls will be freed

;              Set bonds immediately after exit.

;

; Can monitor via internal variables:

;    _pc_n_float  ; Number of floaters

;    _pc_n_active ; Number of floaters being adjusted

;    _pc_max_act_force    ; Max floater force

;    _pc_av_contact_force ; Average contact force

;

; INPUT: flt_remain - proportion of total balls that can be floaters

;        flt_def    - balls with < [flt_def] contacts are "floaters"

;

;--- parameters ---

_pc_r_mult = 1.3  ; Initial radius multiplier

_pc_f_tol  = 0.1  ; Fraction of mean contact force to aim for

_pc_ncut   = flt_def  ; Number contacts to qualify as a floater

_pc_relax  = 1.5  ; ... account for Kn in series

_pc_hyst   = 0.9  ; Hysteresis factor on force reduction

;------------------

_pc_remain = int(flt_remain * ball.num)

;

io.out('_pc_n_float          = ' + string(_pc_n_float         )) ; Number of floaters (internal to this alg.)

io.out('_pc_n_active         = ' + string(_pc_n_active        )) ; Number of floaters being adjusted

io.out('_pc_max_act_force    = ' + string(_pc_max_act_force   )) ; Max floater force

io.out('_pc_av_contact_force = ' + string(_pc_av_contact_force)) ; Average contact force

;command

;  history reset             ; remove _tm_isostr from history list,

;                            ;   it should not be being sampled now

;  hist _pc_n_float          ; Number of floaters (internal to this alg.)

;  hist _pc_n_active         ; Number of floaters being adjusted

;  hist _pc_max_act_force    ; Max floater force

;  hist _pc_av_contact_force ; Average contact force

;end_command

_pc_old_count = 0

loop _pc_outer (1,10)

if flt_num <= _pc_remain then

_pc_rif

exit

endif

_pc_flag_floaters  ; flag and expand floaters

flt_item

if _pc_outer > 1 then  ; skip this check on first pass

if _pc_n_float >= _pc_old_count ; (got itself into a corner)

_pc_ncut = math.max(2,_pc_ncut-1)

flt_def = _pc_ncut

endif

endif

command

cyc 200 ; get clusters to equilibrium

endCommand

section

loop _pc_n (1,100)

_pc_shrink

if _debug = 1 then

_s_flt = string(_pc_n_float)

_s_alt = string(_pc_n_active)

oo = io.out(' floaters: '+_s_flt+' active: '+_s_alt)

endif

if _pc_n_active = 0

_pc_old_count = _pc_n_float

exit section

endif

command

cycle 100

endCommand

endLoop

endSection

_pc_rif

endLoop

_pc_rif

end

; ===================================================================

def flt_num

;

; ----- Returns number of floaters (balls with < [flt_def] contacts)

;       in model.

;

; INPUT:  flt_def - balls with < [flt_def] contacts are "floaters"

; OUTPUT: flt_num - number of floaters

;

flt_num_ = 0

loop foreach bp ball.list

md_numcon_bp = map.size(ball.contactmap(bp))

if md_numcon_bp < flt_def then

flt_num_ = flt_num_ + 1

endif

end_loop

flt_num = flt_num_

end

; ===================================================================

def flt_item

;plot_item

;

; ----- Generates a plotitem for visualizing the floaters.

;

; Visualization Mapping:

;    Floaters are outlined by filled circles that use color(0).

;

loop foreach bp ball.list

md_numcon_bp = map.size(ball.contactmap(bp))

if md_numcon_bp < flt_def then

usp = user.scalar.create(ball.pos(bp))

user.scalar.value(usp) = ball.radius(bp)

endif

end_loop

end

;

; ===================================================================

; PRIVATE FUNCTIONS

; ===================================================================

;

def _pc_rif

; remove inhibit flags

loop foreach cp contact.list

contact.inhibit(cp) = false

endLoop

loop foreach bp ball.list

caseof global.dim

ball.fix(bp, 1) = 0

ball.fix(bp, 2) = 0

ball.fix(bp, 3) = 0

case 3

ball.fix(bp, 1) = 0

ball.fix(bp, 2) = 0

ball.fix(bp, 3) = 0

ball.fix(bp, 4) = 0

ball.fix(bp, 5) = 0

ball.fix(bp, 6) = 0

endcase

endloop

;command

;    ball free velocity spin

;endcommand

end

; ===================================================================

def _pc_flag_floaters ; Flag floaters & expand

;

; ... get mean contact force

_pc_mcf  = 0.0

loop foreach cp contact.list

_pc_mcf  = _pc_mcf + math.abs(contact.force.normal(cp))

endLoop

_pc_n_cf = map.size(ball.contactmap(bp))

if _pc_n_cf > 0

_pc_mcf = _pc_mcf / float(_pc_n_cf)

endif

_pc_av_contact_force = _pc_mcf

_pc_target           = _pc_f_tol * _pc_mcf

_pc_n_float          = 0

; ... identify floaters

loop foreach bp ball.list

ball.extra(bp, 1) = 0

caseof global.dim

ball.vel(bp, 1) = 0.0

ball.vel(bp, 2) = 0.0

ball.spin(bp, 3) = 0.0

ball.fix(bp, 1) = 0

ball.fix(bp, 2) = 0

ball.fix(bp, 3) = 0

case 3

ball.vel(bp, 1) = 0.0

ball.vel(bp, 2) = 0.0

ball.vel(bp, 3) = 0.0

ball.spin(bp, 1) = 0.0

ball.spin(bp, 2) = 0.0

ball.spin(bp, 3) = 0.0

ball.fix(bp, 1) = 0

ball.fix(bp, 2) = 0

ball.fix(bp, 3) = 0

ball.fix(bp, 4) = 0

ball.fix(bp, 5) = 0

ball.fix(bp, 6) = 0

endcase

_pc_count = map.size(ball.contactmap(bp))

if _pc_count < _pc_ncut

ball.extra(bp, 1) = 1

ball.prop(bp, 'fric')  = 0.0

ball.radius(bp)   = ball.radius(bp) * _pc_r_mult

_pc_n_float = _pc_n_float + 1

else

caseof global.dim

case 2

ball.fix(bp, 1) = 1

ball.fix(bp, 2) = 1

ball.fix(bp, 3) = 1

case 3

ball.fix(bp, 1) = 1

ball.fix(bp, 2) = 1

ball.fix(bp, 3) = 1

ball.fix(bp, 4) = 1

ball.fix(bp, 5) = 1

ball.fix(bp, 6) = 1

endcase

endif

endLoop

;-  -- set inhibit flags ---

loop foreach cp contact.list

b1 = contact.end1(cp)

b2 = contact.end2(cp)

if type.pointer(b2) = 'ball-ball' then  ; ball-ball contact

caseof global.dim

case 2

if ball.fix(b1, 3) = 1

if ball.fix(b2, 3) = 1

contact.inhibit(cp) = true

endif

endif

case 3

if ball.fix(b1, 4) = 1

if ball.fix(b2, 4) = 1

contact.inhibit(cp) = true

endif

endif

endcase

endif

endLoop

end

; ===================================================================

def _pc_shrink ; shrink active balls

_pc_max_act_force = 0.0

_pc_n_active      = 0

loop foreach bp ball.list

if ball.extra(bp, 1) = 1

_pc_count = 0

_pc_sum   = 0.0

loop foreach cp ball.contactmap(bp)

_pc_sum   = _pc_sum + contact.force.normal(cp)

endLoop

_pc_count = map.size(ball.contactmap(bp))

if _pc_count > 1

_pc_sum = _pc_sum / float(_pc_count)

if _pc_sum > _pc_target

contact_kn_max = 0.0

loop foreach cp ball.contactmap(bp)

contact_kn_max = math.max(contact_kn_max, contact.prop(cp, 'kn'))

endloop

_pc_delta_r  = _pc_relax * (_pc_sum - _pc_hyst * _pc_target) / contact_kn_max ;ball.prop(bp, 'kn')

ball.radius(bp)    = ball.radius(bp) - _pc_delta_r

_pc_n_active = _pc_n_active + 1

endif

endif

endif

_pc_max_act_force = math.max(_pc_max_act_force,_pc_sum)

endLoop

end

; ===================================================================

return

; END OF Filename: flt.FIS

### dom.p2dat

def get_dom_mem ; ָָ

pnt =  memory.create(dom_size)

if pnt = null

;;ڴʧʾڴ治á

error = 'memory unavailable for domain item'

endif

memory(pnt)    = dom_head  ;

dom_head    = pnt

memory(pnt+1)  = null

get_dom_mem = pnt

end

def dom_scan ;(֮Ŀ϶б)

;--- ĿĿ ---

dom_size = 8

;--- Ŀƫ ...

DOM_LINK      = 0 ;  洢ַ

DOM_BALL_LIST = 1 ;  ָб

;  (˫ȶб (LINK,B_POINT) )

DOM_X         = 2 ;  X(Զ)

DOM_Y         = 3 ;  Y

DOM_PRESS     = 4 ;  ѹ

DOM_VSUM      = 5 ;

DOM_VOL       = 6 ;  ǰ

DOM_FIX       = 7 ;  Լ־ (=1 ̶ѹ)

dom_head      = null

;--- Number of items in flow object (extension to contact) ---

flow_size     = 5

FLOW_DOM1     = 1 ;  ָ domain 1

FLOW_DOM2     = 2 ;  ָ domain 2

FLOW_AP_ZERO  = 3 ;  ϶

FLOW_PERM     = 4 ;  ͸

FLOW_ACTIVE   = 5 ;  = 1 ܵ; else 0

;--- Symbols for domain offsets ...

;--- (domains scan won't work if dead ends exist)

zap_dead_ends

;--- Initialize container to store domain links ---

;    #1 = D pointer corresponding to path B1->B2

;    #2 = D pointer corresponding to path B2->B1

n=0

loop foreach local cp contact.list

contact.extra(cp,FLOW_DOM1)    = null

contact.extra(cp,FLOW_DOM2)    = null

contact.extra(cp,FLOW_AP_ZERO) = 0.0

contact.extra(cp,FLOW_PERM)    = 0.0

contact.extra(cp,FLOW_ACTIVE)  = 0

n=n+1

endloop

scan_all_contacts ; (do twice, to get both senses)

scan_all_contacts

pnt = dom_head    ; Mark the outer domain

count_max    = 0

outer_domain = null

loop while pnt # null

count = 0

bp = memory(pnt+1)

loop while bp # null

count = count + 1

bp    = memory(bp)

endLoop

if count > count_max

count_max = count

outer_domain = pnt

endif

pnt = memory(pnt)

endLoop

oo = io.out('Outer domain has '+string(count_max)+' balls')

end

def domains ; Domain printout

count = 0

pnt = dom_head

loop while pnt # null

if pnt # outer_domain

count = count + 1

oo=io.out(' Domain '+string(count)+'.  Balls are ...')

iadd = memory(pnt+1)

loop while iadd # null

oo = io.out('      '+string(ball.id(memory(iadd+1))))

iadd = memory(iadd)

endloop

endif

pnt = memory(pnt)

endloop

domains = count

end

def scan_all_contacts

;--- Scan contacts, to form domains ---

cpstart = contact_head

loop foreach cpstart contact.list('ball-ball')  ;;;

section

cp = cpstart

b1 = contact.end1(cp)   ;;

b2 = contact.end2(cp)   ;;

c1=contact.extra(cp,FLOW_DOM1)

c2=contact.extra(cp,FLOW_DOM2)

if contact.extra(cp,FLOW_DOM2) # null  ;

if contact.extra(cp,FLOW_DOM1) # null ;

exit section     ;

else                ;

bstart = b2      ;

va1 = contact.normal.x(cp)

va2 = contact.normal.y(cp)  ;

dom_pnt = get_dom_mem       ;

contact.extra(cp,FLOW_DOM1) = dom_pnt  ;

endif

else           ;

bstart = b1     ;

va1 = -contact.normal.x(cp)  ;

va2 = -contact.normal.y(cp)

dom_pnt = get_dom_mem

contact.extra(cp,FLOW_DOM2) = dom_pnt  ;

endif

b2 = bstart

loop nnn (1,1000)      ; (hope outer domain < 1000 balls)

;--- scan contacts on next ball for max angle

max_angle = -1e20

cp_next   = null

next_ball = null

loop foreach _cp ball.contactmap(b2)   ;

_b1 = contact.end1(_cp)

_b2 = contact.end2(_cp)

nn=contact.active(_cp)

if _cp # cp             ;

if _b1 = b2           ;

vb1  = contact.normal.x(_cp)

vb2  = contact.normal.y(_cp)

next_poss = _b2     ;

flag = 1

else

vb1  = -contact.normal.x(_cp)

vb2  = -contact.normal.y(_cp)

next_poss = _b1

flag = 0

endif

cc   = va1 * vb2 - va2 * vb1   ; dot product

maga = math.sqrt(va1*va1 + va2*va2)

na1 = va1 / maga

na2 = va2 / maga

bb  = vb1 * na1 + vb2 * na2    ;

dthet = math.atan2(math.abs(cc),bb^2)    ;

if cc > 0.0

if bb < 0.0

dthet = math.pi - dthet

endif

else

if bb > 0.0

dthet = - dthet

else

dthet = dthet - math.pi

endif

endif

if dthet > max_angle

max_angle = dthet

cp_next   = _cp

next_ball = next_poss

v1sav = vb1

v2sav = vb2

flagsav = flag

endif

endif

endLoop

if cp_next # null

if flagsav = 1

contact.extra(cp_next,FLOW_DOM1) = dom_pnt

else

contact.extra(cp_next,FLOW_DOM2) = dom_pnt

endif

add_ball_to_domain

if next_ball = bstart

exit section

endif

b2   = next_ball

cp   = cp_next

va1  = v1sav

va2  = v2sav

endif

end_loop

;--- end of main contact scan ---

endsection

endLoop

end

def zap_dead_ends

;--- Eliminate balls with one or less contacts ---

num_zapped = 0

loop foreach local ball ball.list

local contactmap = ball.contactmap(ball)

local size = map.size(contactmap)

if size <= 1 then

num_zapped = num_zapped +1

ball.delete(ball)

endif

endloop

if num_zapped > 0

oo =io.out('Number of balls removed = '+string(num_zapped))

else

exit

endif

end

def add_ball_to_domain

;INPUT: dom_pnt, next_ball

pnt = memory.create(2)

if pnt = null

error = 'no memory for ball double'

endif

memory(pnt) = memory(dom_pnt+1)

memory(dom_pnt+1) = pnt

memory(pnt+1) = next_ball

end

def set_dom_coords ; set coordinates for domain centers

; INPUT: dom = domain pointer

iadd = memory(dom+DOM_BALL_LIST)

xav   = 0.0

yav   = 0.0

count = 0

loop while iadd # null

bp    = memory(iadd+1)

xav   = xav + ball.pos.x(bp)

yav   = yav + ball.pos.y(bp)

count = count + 1

iadd = memory(iadd)

endloop

xav = xav / count

yav = yav / count

memory(dom+DOM_X) = xav ; side-effect!

memory(dom+DOM_Y) = yav

end

def dom_item

array vec(2) v1(2) v2(2)

command

set echo off

endcommand

rmin000=100000.

loop foreach bp ball.list

rr=ball.radius(bp)

if rr<rmin000

rmin000=rr

endif

endloop

num_d=0

pnt=dom_head

loop while pnt # null

if pnt # outer_domain

num_d=num_d+1

dom=pnt

set_dom_coords

vec(1)=memory(pnt+dom_x)

vec(2)=memory(pnt+dom_y)

command

geometry generate circle position [vector(vec)] radius [rmin000*0.4]

endcommand

oo =io.out('Number of domains = '+string(num_d))

endif

pnt=memory(pnt)

endloop

oo =io.out('Number of domains last= '+string(num_d))

loop foreach cp contact.list

b1=contact.end1(cp)

b2=contact.end2(cp)

v1(1)=ball.pos.x(b1)

v1(2)=ball.pos.y(b1)

v2(1)=ball.pos.x(b2)

v2(2)=ball.pos.y(b2)

command

geometry edge [vector(v1)] [vector(v2)] group 'contact'

endcommand

if contact.extra(cp,flow_active) = 1

dom1=contact.extra(cp,flow_dom1)

dom2=contact.extra(cp,flow_dom2)

command

list @dom1 @dom2

endcommand

if dom1 # null

v1(1)=memory(dom1+2)

v1(2)=memory(dom1+3)

v2(1)=memory(dom2+2)

v2(2)=memory(dom2+3)

oo=io.out(string(v1(1))+string(v1(2))+string(v2(1))+string(v2(2)))

command

geometry edge [vector(v1)] [vector(v2)] group 'domain'

endcommand

endif

endif

endloop

command

set echo on

endcommand

end

def set_active_flag ; ... on contacts for which flow is calculated

loop foreach cp contact.list

dom1 = contact.extra(cp,FLOW_DOM1)

dom2 = contact.extra(cp,FLOW_DOM2)

if dom1 # null

if dom2 # null

if dom1 # outer_domain

if dom2 # outer_domain

contact.extra(cp,FLOW_ACTIVE) = 1

endif

endif

endif

endif

endloop

end

@dom_scan

@set_active_flag

@dom_item

save dom

### dom1.p2dat

set echo off

[press_max = 0.0]

def pressure

array pvec(2)

;洢άϢ

;ѹƳɾвͬ뾶Բ

;ȣѹ

command

scalar delete ;ɾ֮ǰıscalar

endcommand

pnt       = dom_head

loop while pnt # null;ϵͳеп

if pnt # outer_domain

press_max = math.max(press_max,memory(pnt+DOM_PRESS))

endif

pnt = memory(pnt)

endloop

if press_max = 0.0

exit

endif

pnt = dom_head

num_domain=0

loop while pnt # null

if pnt # outer_domain

dom = pnt

set_dom_coords

vec(1) = memory(pnt+DOM_X)

vec(2) = memory(pnt+DOM_Y)

rad = 0.004 * memory(pnt+DOM_PRESS) / press_max

if rad > 0.0001

vvvv=vector(vec(1),vec(2))

command

scalar create @vvvv @rad

endcommand

endif

num_domain=num_domain+1

endif

pnt = memory(pnt)

endloop

end

def flow_props ;

loop foreach cp contact.list

if contact.extra(cp,FLOW_DOM1) # null

contact.extra(cp,FLOW_AP_ZERO) = ap_zero

contact.extra(cp,FLOW_PERM)    = perm

endif

endLoop

end

def flow_run ;

whilestepping

dddtime=mech.age-time111

if dddtime < flow_dt;flow_dtʱ

exit

endif

time111=mech.age

;  settime0=mech.age

; n_rep = n_rep + 1

;  if n_rep < 10

;    exit

;  endif

;  n_rep=0

;--- Flow in pipes ...

m=0

loop foreach cp contact.list

if contact.extra(cp,FLOW_ACTIVE) = 1

m=m+1

b1    = contact.end1(cp)

b2    = contact.end2(cp)

rsum  = ball.radius(b1) + ball.radius(b2)

dom1  = contact.extra(cp,FLOW_DOM1)

dom2  = contact.extra(cp,FLOW_DOM2)

pdiff = memory(dom1+DOM_PRESS) - memory(dom2+DOM_PRESS)

per_fac = contact.extra(cp,FLOW_PERM)

fnorm = contact.force.normal(cp)

aper0 = contact.extra(cp,FLOW_AP_ZERO)

if fnorm > 0.0

aper = aper0 * Fap_zero / (fnorm + Fap_zero)

else

if gap_mul = 0.0

aper = aper0

else

xdif = ball.pos.x(b1) - ball.pos.x(b2)

ydif = ball.pos.y(b1) - ball.pos.y(b2)

gap  = math.sqrt(xdif*xdif+ydif*ydif) - ball.radius(b1) - ball.radius(b2)

aper = aper0 + gap_mul * gap

endif

endif

qpipe = pdiff * per_fac * aper^3 / rsum

dvol  = qpipe * flow_dt

;command

;list @qpipe @dvol @aper0 @rsum  @fnorm @cp

;endcommand

memory(dom1+DOM_VSUM) = memory(dom1+DOM_VSUM) - dvol

memory(dom2+DOM_VSUM) = memory(dom2+DOM_VSUM) + dvol

endif

endLoop

;ѹ

dom = dom_head

loop while dom # null

if dom # outer_domain

if memory(dom+DOM_FIX) = 0

xdom = memory(dom+DOM_X)

ydom = memory(dom+DOM_Y)

delta_p = memory(dom+DOM_VSUM) * bulk_w ; assume vol = 1

memory(dom+DOM_PRESS) = memory(dom+DOM_PRESS) + delta_p

endif

endif

memory(dom+DOM_VSUM) = 0.0

dom = memory(dom)

endLoop

pressure

;ѹʩӵ

loop foreach bp ball.list

ball.force.app.x(bp) = 0.0

ball.force.app.y(bp) = 0.0

endloop

dom = dom_head

loop while dom # null

if dom # outer_domain

ppp = memory(dom+DOM_PRESS)

iadd = memory(dom+DOM_BALL_LIST)

bstart = memory(iadd+1)

loop while iadd # null

next = memory(iadd)

Abp  = memory(iadd+1)

if next = null

Bbp = bstart

else

Bbp = memory(next+1)

endif

if abp #null

if Bbp # null

Arad = ball.radius(Abp)

Brad = ball.radius(Bbp)

dpr = ppp / (Arad + Brad)

f1 = (ball.pos.y(Abp) - ball.pos.y(Bbp)) * dpr

f2 = (ball.pos.x(Bbp) - ball.pos.x(Abp)) * dpr

ball.force.app.x(Abp) = ball.force.app.x(Abp) + f1 * Arad

ball.force.app.y(Abp) = ball.force.app.y(Abp) + f2 * Arad

ball.force.app.x(Bbp) = ball.force.app.x(Bbp) + f1 * Brad

ball.force.app.y(Bbp) = ball.force.app.y(Bbp) + f2 * Brad

endif

endif

iadd = next

endloop

endif

dom = memory(dom)

endLoop

end

def flow_bc  ;߽

; Range specified with (x1_bc .. x2_bc) and (y1_bc .. y2_bc)

; flow_set:  1  .. fix pressure

;            2  .. free pressure

;            3  .. set pressure to p_given

dom = dom_head

loop while dom # null

if dom # outer_domain

set_dom_coords

xdom = memory(dom+DOM_X)

ydom = memory(dom+DOM_Y)

if xdom > x1_bc

if xdom < x2_bc

if ydom > y1_bc

if ydom < y2_bc

caseOf flow_set

case 1

memory(dom+DOM_FIX) = 1

case 2

memory(dom+DOM_FIX) = 0

case 3

memory(dom+DOM_PRESS) = p_given

endCase

endif

endif

endif

endif

endif

dom = memory(dom)

endLoop

end

def flow_bc_circle(xc0,yc0,rr0) ;߽

; Range specified with (x1_bc .. x2_bc) and (y1_bc .. y2_bc)

; flow_set:  1  .. fix pressure

;            2  .. free pressure

;            3  .. set pressure to p_given

dom = dom_head

loop while dom # null

if dom # outer_domain

set_dom_coords

xdom = memory(dom+DOM_X)

ydom = memory(dom+DOM_Y)

ddd=math.sqrt((xdom-xc0)^2+(ydom-yc0)^2)

if ddd < rr0

caseOf flow_set

case 1

memory(dom+DOM_FIX) = 1

case 2

memory(dom+DOM_FIX) = 0

case 3

memory(dom+DOM_PRESS) = p_given

endCase

endif

endif

dom = memory(dom)

endLoop

end

set echo on

save dom1